# CIVE 202 Project 5 Report: Geotechnical-02

University of Nebraska-Lincoln

CIVE 202 Geost-02-01

Jaycee Aldrich, Sydney Bird, Emma Robbins, Lillian Strohl

May 1, 2026

# Introduction

## Project Summary

The Nebraska Department of Transportation (NDOT) is transitioning its asphalt mixture approval process to a performance-based Balanced Mix Design (BMD) framework. Unlike traditional mix design methods that rely primarily on volumetric properties, the BMD approach requires analysis of large volumes of raw performance test data, including cracking resistance and rutting resistance measurements. This shift significantly increases the complexity and data-processing demands placed on engineers.

To support this transition and speed up the asphalt mixture approval process, our team was tasked with developing a Python-based software application that fully automates the BMD evaluation workflow. The objective of this project was to replace the existing manual Excel-based process with a modular, scalable, and repeatable computational flow.

# By designing the application to be modular and well-documented, the tool ensures reproducibility, minimizes human error, improves efficiency, and provides NDOT engineers with a scalable solution capable of handling large datasets without manual intervention. This automation aligns with modern data-driven engineering practices and strengthens the reliability and consistency of pavement mix approval decisions.

# Project Goals

The project goal is to transition the Nebraska Department of Transportation’s asphalt mixture process to a balanced mix design. Basically, automating their Excel process, making it so that the engineers don’t have to manually take data from their spreadsheets and have excel to the calculations, but take their data, input it into Python, and then have Python do their calculations.

# Project Tasks

Task 1:  Data Collection and Organization of Code

Before any code rewriting can be done, the team needs to read over the given Excel file and raw data files to understand what needs coding. Extracting the data essential to the new code's functionality review the provided Excel file and raw data files to understand what needs to be coded tebook is the first step to transitioning the data into code. The pandas package in Python is necessary for the vitality of the new code. This step is crucial to the success of our code in the coming tasks and must not be skipped over.

Task 2: Develop Aggregate Gradation Automator and Volumetric Analysis

The first part of the code to be developed is for Module A to see the individual aggregate sieve percentages and blending ratios from the raw data. With this data in the code, we will be able to visualize the combined gradation curves and compare them to the NDOT control points. These are needed to verify if the mixture input is within the range of what is needed for the mix design. After Module A is coded, Module B can be created based on the Excel sheet and transferred into the Python notebook. Module B will have equations for volumetric properties and parameters for what the Air Voids must be valid for testing.

Task 3: Performance Testing and Automation

This task is for automating Module C: IDEAL-CT and D: Rutting Analyzer (HWTT). For Module C, code and algorithms need to be written to find an area under a load vs its displacement curve. With this information, we can calculate the post-peak slope and the displacement of 75% of the peak load. These should give us a clean plot of load vs displacement. For Module D, we need to create another algorithm to find the Stripping Inflection Point, which is where the slope of deformation increases drastically because of moisture damage. We will use this algorithm to determine the number of cycles where rut depth passes the 12.5 mm threshold. Another plot of this data is needed for Module D, along with a “Pass/Fail” explanation.

Task 4: Results and Project Documentation

After all the modules have been coded and examined as a team, we will go over the code and make sure the results it is returning to us are correct calculations and are functional for all parties that may use the code. A summary and an ACD will be made available for the code to give everyone an understanding of how the code is used and why it works. All technical and engineering jargon will be transitioned into layman's terms. For NDOT, there will be a user guide that will explain what should be input into the code to get accurate and correct calculations.

# Project Deliverables

The project’s deliverables will be complete based on the request from the client. Through the work of the team, every part of the project will be completed on time. After the “Auto-BMD” code document is completed, there will be a user guide for the Nebraska Department of Transportation to understand the code document. On top of the user guide and code, the team will create a fully functional GitHub repository containing several files relevant to the project for the NDOT to access. The repository will include the code that runs an aggregate gradation automator, volumetric properties calculator, IDEAL-CT, and rutting Analyzer (HWTT). The repository will include an annotated code document explaining the Excel to Python conversions line by line, a comprehensive README with a user guide, and a Gantt chart outlining the project background, methods, results, discussions, and references in accordance with NDOT standards. And finally, a flow chart with how the code works.

| Deliverable | Due to the Client by |
| --- | --- |
| Gantt Chart | 3/6/26 & 5/8/26 |
| Scope of Work | 5/8/26 |
| Python Code | 5/8/26 |
| Annotated Code Document | 5/8/26 |
| Written Technical Report | 5/8/26 *check-ins periodically* |
| User Guide | 5/8/26 |
| Flow Diagram | 3/13/26 &5/8/26 |

# Methods

The development of the Auto-BMD application was divided into four primary function modules, each designed to automate a specific state of asphalt  mixture analysis and performance testing. The application was built using Python, leveraging data processing libraries to handle raw data sets from the Nebraska Department of Transportation. The first module was aggregate gradation automation, this focuses on the mechanical blending of various aggregate stockpiles. The system we designed reads individual sieve percentages and user-defined blending ratios. Then the application calculates the combined gradation curve by applying the blend percentages to each sieve size. The resulting curve is programmatically compared against NDOT control points, these are minimum and maximum specifications. The output includes a visualization of the sieve size vs % passing to ensure the mixture stays within the required specification envelope. The next module is the volumetric properties calculation, this automates the calculation of air voids and other critical mix properties. Using measurements from the volumetrics data tab, the system implements standard volumetric equations. A logic gate was established to flag samples based on Air Void content. If the calculated void falls outside the 6.5% to 7.5% range, the system automatically triggers a warning label. The third module is IDEAL-CT analysis, this evaluates intermediate-temperature cracking resistance, the IDEAL-CT module processes high frequency time-series data. The software identifies the peak load and calculates the area under the load vs displacement curve. Algorithms were written to determine the post-peak slope and displacement at 75% of the peak load. These values are then used to calculate the final CT index for each sample. The final model is the rutting analyzer, this analyses moisture susceptibility and permanent deformation using data from the HWTT. An algorithm was developed to identify the SIP, defined as the point where the deformation slope abruptly increases, signaling moisture damage. The system tracks rut depth over 20,000 cycles. A pass/fail status is assigned based in whether the sample maintains a rut depth of less than 12.5 mm throughout the test duration. The source code was developed as a modular .ipynb file to ensure sequential execution. This modularity allows for easier troubleshooting and future updates to specific testing standards without rewriting the core code.

# Results & Discussion

For the first module the aggregate gradation Automator processed five individual stockpiles ¾ Qtz chips, 3/8 down Qtz, Qtz man sand, akron sand and RAP. The Python logic successfully calculated the combined gradation curve by applying the user-defined blend percentages to the raw sieve data. As shown in the generated Gradation Curve, the combined blend line remains consistently within the grey shaded specification band. The curve follows a well graded distribution, ensuring the mix provides adequate structural stability while maintaining enough void space for binder absorption. The inclusion of RAP was handled correctly by the code, showing its contribution to the finer sieve sizes. The Volumetric properties calculator processed the data from the volumetrics tab to determine the physical characteristics of the compacted specimens. The automated logic checked each sample against the NDOT criteria. Based on the summary output, all processed samples fell within the 6.5 to 7.5% range. Achieving the target air void range is critical for performance. Samples in this range are neither too dense not too porous, justifying their progressions to the IDEAL-Ct and HWTT performance testing stages. For the third stage the IDEAL-CT module analyzed the load vs displacement relationship to determine the cracking tolerance index. The plot illustrates the energy dissipation of the samples. The code successfully identified the peak load and the post-peak slope. The calculated CT index serves as a proxy for intermediate temperature fatigue cracking. A higher CT index indicates a more ductile mix. The results showed a consistent decay curve after the peak load, suggesting that the binder-aggregate bond is sufficient to resist rapid crack propagation. For the final module the rutting analyzer evaluated the Hamburg wheel tracking test data to identify the SIP and total deformation. The mix survived the full 20,000 cycles without exceeding the 12.5mm rut depth threshold. The SIP detection algorithm didn’t find a significant elbow in the deformation curve, indication the mix is resistant to moisture induced stripping. The combination of strong aggregate skeleton and proper volumetrics resulted in a pass status for the HWTT, suggesting high durability under heavy traffic loading.

# Reference List

- Harris, C. R., et al. (2020). Array programming with NumPy. Nature, 585(7825), 357–362. (Used for trapezoidal integration and matrix operations).

- Hunter, J. D. (2007). Matplotlib: A 2D Graphics Environment. Computing in Science & Engineering, 9(3), 90–95. (Used for Gradation and IDEAL-CT plotting).

- The Pandas Development Team. (2026). pandas-dev/pandas: Pandas 2.2.2. Zenodo. (Used for data extraction from Excel and CSV formats).

- AASHTO T 166. Standard Method of Test for Bulk Specific Gravity (Gmb) of Compacted Asphalt Mixtures Using Specimens. American Association of State Highway and Transportation Officials.

- AASHTO T 209. Standard Method of Test for Theoretical Maximum Specific Gravity (Gmm) and Density of Hot Mix Asphalt (HMA). American Association of State Highway and Transportation Officials.

- ASTM D8225-19. Standard Test Method for Determination of Cracking Tolerance Index of Compacted Asphalt Mixture Using the Indirect Tensile Cracking Test at Intermediate Temperature. ASTM International.

- Nebraska Department of Transportation (NDOT). Standard Specifications for Highway Construction. Section 1028: Superpave Asphaltic Concrete.