| Code: | Explanation: |
| --- | --- |
| import pandas as pd | Load pandas – used to read Excel files and build tables |
| import numpy as np | Load numpy – used for math arrays and calculations |
| import matplotlib.pyplot as plt | Load matplotlib – used to draw all the charts |
| xls = pd.ExcelFile("AutoBMD.xlsx") | Open the Excel workbook so we can peek at its sheet names |
| file_path = "AutoBMD.xlsx" print(xls.sheet_names) | Save the file path so we can reuse it across all modules |
| Module A |  |
| df = pd.read_excel(file_path, sheet_name = 'Aggregate Gradation', header=3) | Read the gradation sheet, row 4 is the header (header=3) |
| df.columns = df.columns.astype(str).str.strip() | Clean up column names – remove extra spaces |
| agg_df = df[~df["Aggregate Type"].astype(str).str.contains("Combined¦Specification", na=False)].copy() | Keep only real aggregate row – drop the combined and spec rows |
| sieve_cols = df.columns[3:] | Columns 4 onward are sieve sies (skip label/percentage columns) |
| blend = pd.to_numeric(agg_df["Percentage"], errors='coerce') / 100 | Convert blend % to decimanl |
| gradation = agg_df[sieve_cols].replace('%', '', regex=True) gradation = gradation.apply(lambda col: pd.to_numeric(col, errors='coerce')) | Convert all sieve passing values to numbers |
| combined = np.dot(blend.values, gradation[sieve_cols].values) | Multiply blend fraction * passing values and sum – gives combined curve |
| spec_rows = df[df["Aggregate Type"].str.contains("specification", case=False, na=False)] | Find the spec band rows |
| min_spec = spec_rows.iloc[0, 2:].astype(float).values | First spec row = lower bound of the gradation band |
| max_spec = spec_rows.iloc[1, 2:].astype(float).values | Second spec row = upper bound of gradation band |
| Module B |  |
| vol_df = pd.read_excel(file_path, sheet_name='Volumetrics', header=0) | Read the Volumetric sheet |
| Gse = Ps / ((100 / Gmm) - (Pb / Gb)) | Effective SG of aggregate – how dense the aggregate is ignoring absorbed binder |
| Pba = 100 * Gb * (Gse - Gsb) / (Gsb * Gse) | Absorbed bind - % of binder soaked inot the aggregate |
| Pbe = Pb - (Pba / 100) * Ps | Effective binder - % of binder actually coating the aggregate surface. |
| Va = 100 * (Gmm - Gmb) / Gmm | Air voids - % of the mix volume that is air (target: 6.5-7.5%) |
| VMA = 100 - (Gmb * Ps) / Gsb | Voids in mineral aggregate – total void space between aggregate particles |
| VFA = 100 * (VMA - Va) / VMA | Voids filled with asphalt – what fraction of VMA is filled with binder |
| if 6.5 <= Va <= 7.5: status = "Valid" else: status = "⚠️ Invalid for Performance Testing" | QC check – air voids must be 6.5-7.5% of the sample to be usable |
| summary_df.to_excel('Volumetrics_Summary.xlsx', index=False) print("Summary table saved to: Volumetrics_Summary.xlsx") | Export the results table to a new excel file. |
| Module C |  |
| peak_idx = np.argmax(load) peak_load = load[peak_idx] peak_disp = disp[peak_idx] | Find the index where load is higher (peak of the curve) |
| Wf = np.trapezoid(load * 1000, disp / 1000) | Fracture energy- area under the load-displacement curve |
| Gf = Wf / ((D_mm / 1000) * (t_mm / 1000)) | Fracture energy density – WF normalized by the fracture area |
| P75 = 0.75 * peak_load | Find 75% of the peak load – this is the reference point for the slope |
| l75 = x0 + (P75 - y0) * (x1 - x0) / (y1 - y0) | Interpolate to find the exact displacement where the load hits P75 |
| m75 = (y1 - y0) / (x1 - x0) m75_abs = abs(m75) | Slope of the curve at the P 75 point (how fast load drops) |
| CT = (t_mm / 62.0) * (Gf / m75_abs) * (l75 / D_mm) | CTindex formula – higher values = more crack-resistant mix. |
| Module D |  |
|  | Smooth the rut depth data to reduce noise before finding SIP |
| slope = np.gradient(rut_smooth, cycles) | Rate of change of rut depth – how fast rutting is increasing |
| slope_change = np.gradient(slope, cycles) | Rate of change of the slope – finds where rutting accelerates |
| idx_sip = idx_candidates[np.argmax(slope_change[idx_candidates])] | SIP = the cycle where slope change is maximum |
| crossed = np.where(rut_depth >= threshold)[0] | Find all points where rut depth exceeds the 12.5 mm failure threshold. |
| fail_cycles = x0 + (threshold - y0) * (x1 - x0) / (y1 - y0) | Interpolate to find exact cycle count when rut depth hit 12.5 mm |
| elif failure_cycles >= survive_cycles: return "Pass" | Pass if the sample survives to 20,000 cycles without hitting the threshold |